GSI Format Package

GSI_E_N_H.FRT

GSI format with output as Point Number, Easting (Target), Northing (Target), Elevation (Target).

11, 81, 82, 83

*110001+00000000PP123-98 81..00+0000000140123877 82..00+0000005123333565 83..00+0000000001000223

### GSI_E_N_H_C.FRT

GSI format with output as Point Number, Easting (Target), Northing (Target), Elevation (Target), Code name.

11, 81, 82, 83, 41

*410001+0000000000Tree01

*110001+00000000PP123-98 81..00+0000000140123877 82..00+0000005123333565 83..00+0000000001000223

GSI_hz_hd_dh.FRT

GSI format with output as Point Number, Horizontal Angle, Horizontal Distance and Height difference.

11, 21, 32, 33

*110001+00000000PP123-98 21.000+0000000032111110 32..00+0000000000580232 33..00+0000000000029232

GSI_hz_hd_dh_c.FRT

GSI format with output as Point Number, Horizontal Angle, Horizontal Distance, Height difference, Easting (Target), Northing (Target), Elevation (Target), Code name, Info1, Info2, Info3, Info4.

11, 21, 32, 33, 81, 82, 83, 41, 42, 43, 44, 45

*410001+0000000000Tree01 42....+000000Attribute1 43....+000000Attribute2 44....+000000Attribute3 45....+000000Attribute4

*110001+00000000PP123-98 21.000+0000000032111110 32..00+0000000000580232 33..00+0000000000029232 81..00+0000000140123877 82..00+0000005123333565 83..00+0000000001000223

GSI_hz_v_enh.FRT

GSI format with output as Point Number, Horizontal Angle, Vertical Angle, Slope Distance, Easting (Target), Northing (Target), Elevation (Target), Target Height and Instrument Height.

11, 21, 22, 31, 81, 82, 83, 87, 88

*110001+00000000PP123-98 21.000+0000000032111110 22.000+0000000008813130 31..00+0000000000609173 81..00+0000000140123877 82..00+0000005123333565 83..00+0000000001000223 87...0+0000000000001300 88...0+0000000000001720

GSI_incl_Station.FRT

GSI format with output of

- Station ID, Easting (Station), Northing (Station) and Elevation (Station),

- Point Number and measurement to Known Backsight,

- Point Number, Horizontal Angle, Vertical Angle, Slope Distance ppm, Reflector height, Easting (Target), Northing (Target) and Elevation (Target).