Leica TPS 300 Total Stations

Data Output Formats for TPS300 Total Stations

Leica’s TPS300 offers unequalled flexibility in data output.  Your TPS300 Total Station can provide raw field data or coordinates in virtually any file format.

The following table illustrates the output formats supported by TPS300.  Be sure to read the information on Units at the end of the table.

|  | # TPS300 Data Output Formats |
| --- | --- |
| # Name | # Sample Output |
| ENH | Space delimited ASCII format: P E N H C I1 I2 I3 I4 |
|  | CP2 10000.000 10005.295 1100.064 Ash 100 10006.510 10006.311 1101.038 Pine 101 10006.100 9998.361 1097.501 Hickory 102 9996.223 9997.183 1096.227 Oak 103 9997.158 10004.840 1095.063 Maple |
| NEH | Similar to ENH but the northing value is reported before easting |
| ENHC | Comma delimited ASCII format: P, E, N, H, C I1 I2 I3 I4 |
|  | CP2,10000.000,10005.295,1100.064,Ash 100,10006.510,10006.311,1101.038,Pine 101,10006.100,9998.361,1097.501,Hickory 102,9996.223,9997.183,1096.227,Oak 103,9997.158,10004.840,1095.063,Maple |
| NEHC | Similar to ENHC but the northing value is reported before easting |
| ENHC2 | Similar to ENHC but attributes are separated with commas |
| NEHC2 | Similar to NEHC but attributes are separated with commas |
| STD8 | Leica GSI8 file format with output of horizontal angle, vertical angle and slope distance. |
|  | 410001+00000Ash 42....+00000000 43....+00000000 44....+00000000 45....+00000000 110002+00000CP2 21.104+03595957 22.104+00870824 31..01+00005302 51..1.+0000+000 410003+0000Pine 42....+00000000 43....+00000000 44....+00000000 45....+00000000 110004+00000100 21.104+00455329 22.104+00771943 31..01+00009294 51..1.+0000+000 410005+0Hickory 42....+00000000 43....+00000000 44....+00000000 45....+00000000 110006+00000101 21.104+01050217 22.104+01032048 31..01+00006491 51..1.+0000+000 |
| REPORT | A report listing all data. |
|  | ***** TPS300 Data File **** Code Info1 Info2 Info3 Info4 Tree01 Attribute1 Attribute2 Attribute3 Attribute4 Station Point H.I. ST123-12 1.7200 Orientation Pt Horz. Angle BsPt 122.4351 Measured Point H.T. Horz. Angle Vert. Angle Slope Dist. PP123-98 1.3000 321.1111 88.1313 609.1730 |
| SDR20 | SOKKIA SDR20 file format. |
|  | 00NMSDR20 V03-05 111111 08KIPP125123333.565140123.877 1000.223 10NMJobname1 13NMOperator: JohnDoe 01NMN 000000 00000031 0.000 03NM 1.300 09F1ST12PP12 609.173 88.1313 321.1111 Attribute1 Attribute2 |
| SDR33 | SOKKIA SDR33 file format. |
|  | 00NMSDR33 V04 111111 08KI PP123-98 5123333.565 140123.877 10NM Jobname1 13NMOperator: JohnDoe 01NMN 000000 00000031 0.000 03NM 1.300 09F1 ST123-12 PP123-98 609.173 88.13130000 321.11110000 |
| EgPt_300 | Eagle Point data format Data forma based on modifed EaglePoint code table and TPS300 codelist |
| ##### FBK | AutoDesk/Softdesk fieldbook format |
|  | STN 1 1.111 NEZ 2 1.00 1.00 1.00 BS 2 0.000 BEG EP1 ! EP PRISM 0.000 F1 VA 1040 219.27570 3.031 68.24202 EP |
| # DXF | # DXF Data Output Formats |
| DXF_1 | Point Number and a Node are drawn. For Fixed Points: on layer named FIXED. For Station points: on layer named STATION. For Measured points: on layer named with the CODE key. |
| DXF_2 | Similar to DXF_1, but Point Number are on layer of suffix _ID, elevations are on a layer of suffix _ELEV and Description on layer of suffix _DESC |
| DXF_3 | Similar to DXF_2, but information is placed around the node. |
|  | Data generated through a program |
| CutSheet | Cut/Fill Sheet Project: Jobname1 Date: 07/02/1998 ------------------------------------------------------------------------- Pt# DesignH ActualH DEast Dnorth Cut-/Fill+ ------------------------------------------------------------------------- PP123-12 1298.25 1000.22 -0.00 0.00 0.02 |
| ##### TieDistance | Tie Distance Computations ========================= From Point: Pt1 To Point: Pt2 Azimuth: 34.1234 H Dist: 19.73 V Dist: 20.92 |
| TieDistance2 | Tie Distance Results ==================== From Point To Point Azimuth Horz. Dist Height Diff ================ ================ ========== ========== ========== Pt1 Pt2 34.1234 19.73 20.92 |

|  | Units |  | Some format comes in two options, one for Metric units and one for US Feet. All angular values are in 360 DDD.mmss If only one file exists, units from instruments will be used. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | The format filenames contain information about the content and units for each format. The last character of the filename designates the units. |  |  |  |  |  |  |
|  |  | Example |  | SDR20_U.FRT. | This file contains the SDR20 format using US Feet. |  |  |  |  |
|  |  |  |  | SDR20_M.FRT | This file contains the SDR20 format using Meters. |  |  |  |  |
| If you don’t see what you need… |  | Contact your Leica representative. We can work with you to create custom formats that fit your needs. |  |  |  |  |  |  |  |